AV20476RS	20MP CONTERAIP OMNI LX RS 4 X 2592X1944 H.265/H.264/M-JPEG, WDR, REMOTE SETUP WITH REMOTE FOCUS , ZOOM, PAN, TILT, , 4 X 3.3-6.6MM MOTORIZED LENS, 15FPS, , AUDIO, IP66, IK-10, ONVIF, POE+/ 12V DC/ 24V AC

This A&E specification is written according to Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat™ (2009 Edition) and The Project Resource Manual – CSI Manual of Practice.     www.csinet.org/masterformat.

Manufacturer is responsible for the accuracy of the technical data included in this specification.

.

Division 28 – Electric Safety and Security

Section 28.23.29 – Video Surveillance – Remote Devices and Sensors

Part 1   General

## 1.1   General Requirements

The camera shall be of manufacturer’s official product line, designed for continuous commercial or industrial use.

The camera shall be based on standard parts and components and utilize proven technology using open and published protocols.

All camera installation, configuration, setup, programming and all related work shall be performed by electronic technicians thoroughly trained in the installation and service of the equipment provided and in complete compliance with all local codes and regulations.

All equipment provided shall be backed by a three-year manufacturer warranty.

## Certifications and Standards

- European Community Directives:

2004/108/EC (EMC Directive);

2006/95/EC (Low Voltage Directive);

2011/65/EU (RoHS Directive)

1907/2006/EC (REACH Directive)

2002/96/EC (WEEE Directive)

- European EMC Standards to which conformity is declared:

EN 55024:2010 + A1:2015

EN 55032:2015 + AC:2016 Class A

EN 55035:2017

EN 61000-3-2:2014

EN 61000-3-3:2013

- UL Listing

CB Test Report (IEC 60950-1 (ed. 2) and IEC 60950-22 (ed. 1)), E489591

- FCC Standard Compliance:

Title 47, Part 15 (47 CFR 15) Subpart B Class A

- Mechanical Standards:

ANSI/IEC 60529-2004 - IP66 dust/water Ingress protection rating

EN62262:2002 – IK-10 impact rating

- Video Compression Technology

H.264 MPEG-4, Part 10 ISO/IEC 14496-10 AVC/H.265

- Networking Standard:

IEEE 802.3at-2009 PoE Standard

## Part 2     Products

2.1    Manufacturer

Arecont Vision Costar, LLC

400 N Brand Boulevard, Suite 860

Glendale, CA 91203-2311 USA

Phone: 818-937-0700

877-226-3728

Fax:      818-937-0464

www.arecontvision.com

2.2    General

The ConteraIP® Omni LX Remote Setup (RS) is an industry-game-changing first-of-its-kind omni-directional, remote-configurable, multi-sensor, multi-megapixel camera built to provide outstanding high resolution video coverage for a wide range of applications. The unmatched coverage and capabilities of the ConteraIP® Omni LX RS provides organizations of all sizes the flexibility to deploy a surveillance camera system that truly matches their current and future requirements for complete situational awareness.

ConteraIP® Omni LX RS is available with a choice of 8- or 20-megapixel (MP) resolutions. The number of cameras required for a project can be dramatically reduced with a single ConteraIP® Omni LX RS, thanks to its four customizable remote sensor gimbals. Installation is fast and easy. Install the hinged mounting plate, connect the PoE+ (Power-over-Ethernet) IP cable, and then remotely configure the camera. Select a preset choice for 180°, 270°, or 360° views, or use the intuitive interface to remotely pan, tilt, zoom, and focus each sensor. Two custom presets created by the user can be saved to memory. The camera is integrated with the industry’s leading VMS/NVR platforms. The microSD card slot supports up to 256GB of storage capacity for convenient onboard storage.

ConteraIP® Omni LX RS is ideal for applications with normal to challenging lighting conditions. The Omni combines a day/night mechanical IR cut filter for the highest image quality at any time of day. For clear color images in low-light, NightView™ offers strong low-light sensitivity for capturing details in extremely poor-lit scenes. Power can be supplied via a single PoE+ (802.3at) compliant network cable or via an 12–48V DC/24V AC power supply.

ConteraIP® Omni LX RS is designed for demanding environments. Certified to rigorous dust and water tests, the camera carries an IP66 rating. The rugged dome housing is IK-10 rated to withstand the equivalent of 55 kg (120 lbs) of force for vandal-prone applications.

Arecont Vision® was the first to bring H.264 to the mainstream market and recently developed SNAPstream™ (Smart Noise Adaptation and Processing) technology for reducing bandwidth without impacting image quality. Today we are proud to offer our next generation H.265 with SNAPstream+™ smart codec capable of delivering high quality video while saving over 50% of the data rate to reduce or prevent strain on the network.

The ConteraIP® Omni LX RS is ONVIF (Open Network Video Interface Forum) Profile S, G, Q and T compliant, providing interoperability between network video products regardless of manufacturer.

2.3   Hardware

- The camera shall utilize four high sensitivity 5.1 Megapixel CMOS sensor with 1/2.8” optical format, 2.0um x 2.0um pixel size, progressive scan and Active Pixel Count: 2592(H) x 1944(V) pixel array.

- The camera shall have and integrated four 3.3-6.6mm megapixel, IR corrected, varifocal focal lens with 1/2.5" optical format, F/1.7 and horizontal field-of-view of 99°- 52°.

- The camera shall have SDHC card slot for onboard storage up to 256GB, class 1- and UHS-1.

- The camera shall have die-cast aluminum chassis with IK-10 vandal resistant dome. Entire enclosure to be rated minimum IP66 for water and dust protection.

- The camera shall have an integrated remote setup platform which offers remotely configurable zoom, focus, pan, and tilt.

- The camera shall have four individually adjustable motorized 2-axis camera gimbals with 360˚ pan and 135˚ tilt for easy and accurate positioning.

- The camera’s power source shall be Power over Ethernet (PoE+) complying with the IEEE 802.3at standard to support IR illuminator and camera.

- The camera shall have an auxiliary power input, AC24V and DC12V.

- The camera shall be utilized for indoor and outdoor applications.

2.4   Imaging

- The camera shall combine four image sensors for a user configurable field of view.

- The camera shall support three predefined camera preset configurations: 180 degree, 270 degree, and 360 degree.

- The camera shall support two custom preset configurations.

- The camera shall have standard compression support with simultaneous streaming of H.265, H.264 and MJPEG formats.

- The camera shall feature automatic exposure, automatic multi-matrix white balance, shutter speed control to minimize motion blur, programmable brightness, saturation, sharpness, contrast and hue.

- The camera’s shutter speed shall be 0.1ms - 500ms.

- The camera shall feature 5-255 Hz flicker control, windowing, simultaneous delivery of full-field view and zoomed images at video frame rate, instantaneous electronic zoom, pan and tilt, and electronic image rotation by 180 degrees.

- The camera shall have multi-streaming support of up to two H.265/ H.264 streams and one M-JPEG stream.

- The camera shall have wide dynamic range up to 100dB and a maximum SNR of 46dB.

- The camera shall have privacy masking, the ability to select multiple regions of an arbitrary shape to block the video.

- The camera shall have Real Time Streaming Protocol (RTSP) support allowing for compatibility with media players such as Apple QuickTime, VLC Player and others.

- The camera shall have extended motion detection grid, a higher granularity grid of 1024 distinct motion detection zones. User can select between 64 zone based motion detection and extended motion detection to provide backward compatibility with the existing Video Management System (VMS) integration.

- The camera shall provide 21 levels of compression quality for optimal viewing and archiving.

- The cameras H.265/ H.264 implementation shall maintain full real time video frame rates.

- The camera shall output at a maximum resolution of 2592 (H) x 1944(V) pixels per channel up to frame rate of 15 frames per second (FPS) of each sensor.

- It shall be possible to program the camera to output a variety of lower resolution image, i.e. 2048(H) x 1536(V) pixels per channel at 15 FPS of each sensor.

- The camera shall be able to save bandwidth & storage by running at 1/4 full resolution, and Bandwidth Limit Control.

- The camera shall be able to have below scaled resolutions per channel: 2560x1440, 2304x1296, 2048x1536, 1920x1080, 1600x1200, 1296x972, 1280x960, 1280x720, 800x600, and 640x480.

- The camera shall feature MoonLight™ mode - extended exposure and noise cancellation

- This camera shall have SNAPstream+™ (Smart Noise Adaptation and Processing) capability to reduce bandwidth without impacting image quality.

- The camera shall have On-Screen Display (OSD).

2.5   Video

Video frame rate (each sensor up to):

15fps @ 4x 2592x1944

15fps @ 4x 2592x1944 + 5fps @ 4x 2592x1944

15fps @ 4x 2592x1944 + 5fps @ 4x 2592x1944 + 15fps @ 4x 1280x960

15fps @ 4x 2048x1536 + 15fps @ 4x 2048x1536 + 15fps @ 4x 640x480

Scaled Resolution:

2560x1440, 2304x1296, 2048x1536, 1920x1080, 1600x1200, 1296x972, 1280x960, 1280x720, 800x600, and 640x480.

2.6   Protocols

- The camera shall have Real Time Streaming Protocol (RTSP) support allowing for compatibility with media players such as Apple QuickTime, VLC Player and others.

- The camera shall support both unicast and multicast communication protocol.

- The camera shall support SNMP, IPv6, IPv4, HTTP, HTTPS, SSL, TCP/IP, UPnP, UDP, RTCP, RTSP, RTP, SMTP, NTP, DHCP, FTP, 802.1x, Zero Configure.

- 100/1000 Base-T Ethernet Network Interface

2.7   Electrical

Alarm Input (Wet Contact) and Alarm Output (Wet Contact)

Power over Ethernet (PoE): PoE+ 802.3at

Auxiliary Power 12V DC, 24VAC

Power consumption: PoE – 23 Watts maximum (with all motors on)

2.8 Audio Electrical

Streaming	Two-way

Compression	G.711 PCM 8kHz

Input/Output	Line in / Line out

2.9   Networking

The camera shall be equipped with a 1000 Mbps LAN connector.

2.10   Environmental

Operating temperature: -40°C (-40°F) to +50°C (122°F)

Storage temperature: -40°C (-40°F) to +60˚C (140 °F)

Humidity 0% to 90% (non-condensing)

2.11   Minimum Illumination

Color (Day Mode): 0.1 Lux

B/W (Night Mode): 0.01 Lux

www.megapixelvideo.com			info@arecontvision.com			© 2005 Arecont Vision

2.12   Packaging

Unit Dimensions (H x Dia) 4.9”H (125 mm) x 8.8” dia. (223 mm)     Weight: 5.2lbs (2.36kg)

Packaged Dimensions (H x W x L) 7.4” (188mm) x 11.1” (281mm) x 11.1” (281mm)   Weight: 5.9lbs (2.68kg)

2.13   Compatible Accessories

AV-1AK		ConteraIP Optional Audio Kit

AV-CRMA	Corner Mount Adapter

AV-CRMA-W	Corner Mount Adapter (White)

AV-JBA	 	Junction Box Adapter

AV-JBA-W 	Junction Box Adapter (White)

AV-PMA		Pole Mount Adapter

AV-PMA-W 	Pole Mount Adapter (White)

AV-PMJB 	Pendant Mount

AV-PMJB-W	Pendant Mount (White)

AV-WMJB 	Wall Mount Bracket

AV-WMJB-W 	Wall Mount Bracket (White)

SO3-FMA	Flush Mount Adapter

SO3-CAP		Mount Cap

SO3-CAP-W	Mount Cap(white)

2.14   Related Documents

ConteraIP® Omni LX RS Datasheet

ConteraIP® Omni LX RS Installation Manual